# Finch2 Library Migration Guide

This document summarizes the key differences between the older Finch library and the updated Finch2 library. Use this as a quick reference when updating lesson materials.

## Setup and Initialization

| Old Library | Finch2 Library |
| --- | --- |
| from finch import Finch bird = Finch() | from finch import Finch bird = Finch() |

No change required

## Motor Control

| Old Library | Finch2 Library |
| --- | --- |
| bird.wheels(1, 1) Speed range: -1 to 1 | bird.setMotors(100, 100) Speed range: -100 to 100 |
| bird.wheels(0, 0) | bird.stop() |

## Beak LED Control

| Old Library | Finch2 Library |
| --- | --- |
| bird.led(255, 0, 0) RGB range: 0 to 255 | bird.setBeak(100, 0, 0) RGB range: 0 to 100 |

## Sound/Buzzer

| Old Library | Finch2 Library |
| --- | --- |
| bird.buzzer(2, 440) Parameters: duration (seconds), frequency (Hz) | bird.playNote(60, 2) Parameters: note number (32-135), beats (0-16) |

## Temperature Sensor

| Old Library | Finch2 Library |
| --- | --- |
| temp = bird.temperature() | temp = bird.getTemperature() |

## Closing Connection

| Old Library | Finch2 Library |
| --- | --- |
| bird.close() Required at end of program | Automatic Connection closes automatically when program ends |

## New Features in Finch2

The Finch2 library includes several convenient new methods that weren't in the old library:

setMove(direction, distance, speed)

Moves the Finch forward or backward for a specific distance. Example: bird.setMove('F', 10, 50) moves forward 10 cm at speed 50.

setTurn(direction, angle, speed)

Turns the Finch right or left for a specific angle. Example: bird.setTurn('R', 90, 50) turns right 90 degrees at speed 50.

setTail(port, red, green, blue)

Controls the four tail LEDs. Example: bird.setTail("all", 100, 0, 0) sets all tail LEDs to red.

stopAll()

Stops all motors, lights, and sounds at once.